DSP Note

Made By Masud Rana Mushfiq

- Analysis of Discrete Time LTI system                         Page: 10

- Z-transform                                              Page: 18

- Fourier Transfrom                                         Page: 23

- DFT                                                    Page: 27

- Filter                                                   Page: 35

- Definition of DSP: Digital Signal Processing deals with the enhancement, extraction, and representation of information for the purposes of communication and analysis.

- Signal: A signal is defined as any physical quantity that varies with time, space, or any other independent variable or variables. Mathematically, we describe a signal as a function of one or more independent variables.

- For example, the functions

- s1(t) = 5t

- s2(t) = 20t2

- System: A physical device or algorithm that performs operations on a signal.

- Signal Channels: Signals can be categorized based on the number of information streams or channel:

- Single Channel: A single stream of data.

- Multichannel: Multiple streams or sensors, such as RGB image data or ECG 			     readings.

- Transducers: Devices that acquire natural signals (like sound or pressure) and convert them to electrical signals (e.g., voltmeters, microphones, pressure sensors).

- Amplitude vs. Magnitude:

- Amplitude: Can be positive or negative.

- Magnitude: Always positive; refers to the absolute value.

To convert an analog signal x(t) to a discrete-time signal x(n)

Three steps are required:

- Sampling: Taking snapshots of the continuous signal at specific time intervals

- Quantization: Mapping continuous amplitude values to a finite set of levels.

- Coding: Assigning unique binary codes to each quantization level.

A typical DSP system contains the following hardware stages:

- Input Lowpass Filter: Used to avoid aliasing by removing unwanted high-frequency components.

- Analog-to-Digital Converter (ADC): Converts the analog input into a digital format.

- DSP Processor: A specialized computer or processor that performs mathematical operations on digital signals.

- Digital-to-Analog Converter (DAC): Converts the processed digital data back into analog form.

- Output Lowpass Filter: Used to avoid imaging (distortion appearing during reconstruction).

Applications of DSP

- Biological Signals: Processing of EEG, ECG, X-ray, PET scans, and MRI.

- Speech Processing: Analysis of Formant frequency (vocal cord frequency).

- Biometrics: Used for personal identification.

- Audio Signals:

    - Active noise cancellation.

    - Digital Audio Effects.

    - Audio signal separation.

- Advantages of DSP over Analog

- Better Accuracy Control: Digital systems provide precise control using word 					    length and numerical methods.

- Easy Storage: Signals can be stored on disk/tape without loss (except minor 			     A/D conversion loss).

- Portability & Offline Processing: Stored signals can be transported and 							       processed later in remote locations.

- Advanced Algorithms: Complex signal processing operations can be easily 					implemented.

- Cost Advantage: In some cases, digital systems are cheaper than analog 					systems.

Signal Classification

- Time Domain Classification:

- Stationary Signals: Statistical properties remain constant over time.

- Non-stationary Signals: Properties change over time (e.g., speech signals).

- Deterministic vs. Random Signals:

- Deterministic: Can be predicted exactly by a mathematical formula.

- Random (Non-deterministic): Cannot be predicted (e.g., noise, speech).

- Signal Types:

- 1. Continuous-Time Signal x(t)

- A continuous-time signal is defined for every value of time.

- Represented as a smooth, continuous waveform

- Time variable t is continuous

- Example: Analog signals like sound waves, voltage signals

- 2. Discrete-Time Signal x[n]

- A discrete-time signal is defined only at specific time instants (point).

- Represented as a sequence of values (spikes/samples)

- Time variable n is an integer (…, -2, -1, 0, 1, 2, …)

- Usually obtained by sampling of a continuous signal

- Discrete Time Signal Values:

- Continual Values: Fractional values can be appear.

- Discrete Value: Only integer values are used.

Continuous Time Signal Representation

- xa(t) = Acos(Ωt + θ).                (− ∞ < t < ∞)

- Variables:

- A: Amplitude.

- Ω (Capital Omega): Frequency in radians/second (In Time Domain).

- Ω = 2πF.

- θ: Phase in radians.

- F: Angular Frequency, Capital for continuous time signal (F = 1/T).

- T: Period time.

Discrete Time Sinusoidal Signal

- Equation: x[n] = Acos(ωn + θ).              (− ∞ < n < ∞)

- Variables:

A: Amplitude

ω (Small omega): ω= 2πf (Angular frequency in radians/sample)

θ: Phase in radians

n: Sample index (Discrete time variable where n = 0,1,2,3,…)

Relationship with normalized frequency: ω = 2πf

f: Small Letter, Frequency in cycles/sample (Normalized Frequency)

N: Fundamental period (number of samples)

f = 1/N​

Normalized frequency is the frequency of a discrete-time signal measured in cycles per sample. It has No unit (dimensionless).

f = F/Fs

Where:

f = normalized frequency (cycles/sample)

F = actual frequency (Hz)

Fs = sampling frequency (samples/sec)

Normalized means:

Because the frequency is scaled (normalized) by the sampling frequency:

- Complex Exponential Representation of signal(Analog): xa(t) = Ae j(Ωt+θ).

- Euler's Identity: e ± jΩ = cosΩ ± jsinΩ.

- Periodicity Rule for Discrete-time signal: A signal is periodic if its frequency (f ) is a rational number (f = k/N), where k and N are integers.

- To Show the above assumption we need to prove ω/ 2π = k/N

- Frequency Uniqueness:

- Discrete-time sinusoids whose frequencies are separated by an integer multiple of 2π are identical .

- Frequencies are distinct (unique) within the range: −π≤ω≤π  or −1/2≤ f≤ 1/2

- Outside this range, the signals are identical (aliases) .

- Signal Periodicity

- Fundamental Period (N): The smallest integer N for which the condition x(n+N) = x(n) is satisfied.

- Composite Period: For a signal composed of multiple sinusoids, the fundamental period is the Least Common Multiple (LCM) of the individual periods.

- Calculations and Examples

- Example 01: (Periodicity of Discrete Signal)

- cos(0.01πn)

- A = 1, ω = 0.01π, θ = 0.

- 2πf = 0.01π

- f =1/200 = k/N.

- Since both are integers, it is rational and therefore it is periodic.

- Example 02: (Periodicity of  Analog Signal)

- xa(t) = 3cos(5t + π/6)

- A = 3, Ω = 5, θ = π/6.

- 2πF = 5 ⟹  F = 5/2π.

- Fundamental Period (Tp) = 1/F = 2π/5.

- Proof of Periodicity: 3cos(5t + π/6) = 3cos[5(t + Tp) + π/6].

- Substituting Tp yields 3cos(5t + 2π + π/6), which is equivalent to the original signal

Periodicity of Composite Signals: x(n) = cos(2πn) − sin(8πn) + 3cos(4πn + 3π)

- Component 1: ω = 2π​⟹  f1​= 2ππ/2​= 1​

- Component 2: ω = 8π​⟹f  2​= 2ππ/8​= 1​

- Component 3: ω = 4π​⟹  f3​= 2ππ/4​= 1​

Conclusion: Then all are rational so all are periodic.

Fundamental Period = LCM(f1 ,f2 ,f3 ) = 1

Ex 03: x(n) = cos(2πn) − sin(8πn) + 3cos(4πn + 3π)

It will work if you do each part separately

Ex 04: 2ej(n/6−π)

Bring this into sin-cos using Euler's Identity and then do it

- Quantization

- Number of Quantization Levels (L): For a b-bit quantizer, L = 2b.

- Quantization Step Size:   Δ = xmax−xminL−1, where xmax and xmin are the range limits.

- And L is the totall number of level.

- Quantization Error (e): e = Q(x) − x, where Q(x) is the quantized value and x is 							  original value.

- Step Value Range of Error: Typically −Δ/2 ≤ e ≤ Δ/2​

- Step Calculation: A 3-bit quantization provides 2^3 = 8 steps.

- Error Relation:

- Lower bit depth: Results in more quantization error/noise .

- Higher bit depth: Results in less quantization error.

- Concepts: Quantization noise, quantization error, and bit depth .

- Energy and Power Signals:

- Energy Signal: A signal is an energy signal if it has finite total energy and zero average power. 0<E<∞, P = 0

- Power Signal: A signal is a power signal if it has finite (non-zero) average power but infinite energy.  0<P<∞, E = ∞

- Energy Signal and Power for Discrete-time Signal:

- Energy Signal (E): E =n = −∞∞∣x(n)2∣.

- Average Signal Power (P): P = limN→∞​1/(2N+1)​∑n=−NN |x(n)|2.

- If a Discrete-time Periodic Signal have fundamental period N then-

- Power :

- P  = 1/N​∑n=0N−1  |x(n)|2.

- Even and Odd Signal Decomposition

- Even (Symmetric) Signal: left and right sides are identical respect to n = 0

- x(n) = x(−n).

- Odd (Antisymmetric) Signal: left and right sides are opposite respect to n = 0

- x(n) = −x(−n).

- Even Component (xe): xe(n) = 12 [x(n) + x(−n)]

- Odd Component (xo): xo(n) =  12 [x(n) − x(−n)]

- Total Signal: x(n) = xe(n) + xo(n)

Transformation of Independent Variables: Transformation of independent variables means modifying the time index n of a signal to shift, flip, or rescale it.

- Time Shift: f(n) = x(n - n0)

- If n0 is positive: Result is a Delay (Shift Right).

- If n0 is negative: Result is an Advance (Shift Left).

- Time Reversal (Folding): Flipped signal, represented as x(-n).

- Time Slicing (Sampling Rate Change):

- Downsampling: f(n) = x(Mn)

- Upsampling: f(n) = x(n/N)

System Classification: Static vs. Dynamic:

- Static System:Memoryless

- Output depends only on the present input (e.g., y(n) = x(n).

- Dynamic System: Has memory

- Output depends on past or future inputs (e.g., y(n) = x2(n) + x(n-2)).

- Time-Invariant vs. Time-Variant:

- A system is called time-invariant if its input–output characteristics do not 			change with time.

- If the input-output characteristic changes over time, it is Time-Variant.

- Causality:

- Causal System: A causal system Output depends on present and past inputs.

- Non-causal System: A non causal system Output depends on future inputs.

- Linear vs. Non-linear:

- Linear System: Must satisfy the principle of superposition (Homogeneity and Additivity).

- Superposition Formula: T[a1x1(n) + a2x2(n)] = a1T[x1(n)] + a2T[x2(n)]

- Breakdown of Superposition

- (a) Homogeneity (Scaling)

- T[a x(n)] = a T[x(n)]

- (b) Additivity

- T[x1(n) + x2(n)] = T[x1(n)] + T[x2(n)]

- Stability:

- A system is Stable if it produces a bounded output for a bounded input (BIBO).

- Example: y(n) = u2(n) is stable because u(n) can be estimated.

- Example: y(t) = intrigation u(t) dt. Here it is (Ramp) so unstable.

- Bounded/Unbounded: If value change or unbounded in y-axis then it is unbounded

- Converging: These signals are bounded.

- Diverging: These signals are unbounded.

- If a signal grows to infinity → unbounded; otherwise (finite/decaying/oscillating) → bounded.

Discrete Time Signal Representation

- Topic: δ(−n)→?

- Basic Signals: We learn unit impulse, unit step, ramp, and exponential basic signals and later we use them to analysis complex signal.

- Complex Analysis: x(n) = an is used to describe complex signals for easier analysis.

- Important Rule: When both fold and shifting exist, it will be the opposite of the normal rule.

- Procedure: First fold then shift.

Impulse and Unit Step Relations

- Unit Step (U(n)): U(n) = δ(n) + δ(n − 1) + δ(n − 2) + …

- Impulse Response: δ(n)

- Formula: δ(n) = U(n) − U(n − 1)

Example: Representation in terms of impulse response

Given a signal graph with values at n = −1, 0, 1, 2:

- x(n) = δ(n + 1) + 2δ(n) + δ(n − 1) + 3δ(n − 2)

Basic Operation:

- Unite Delay: x(n) → [z −1] this will be  y(n) = x(n − 1)

- Sum (∑), multiplier (×), scalling (α): These are the core mathematical operations applied to signals.

- Advance: x(n) → [z] this will be y(n) = x(n + 1)

Example Diagram and Difference Equation:

The notes provide a diagram illustrating how these operations are combined to form a system.

From this diagram, a Difference equation is derived:

y(n) = 0.5x(n − 1) + 0.5x(n) + 0.25y(n − 1)

- 0.5x(n − 1): Represents the input signal with a unit delay (z−1) and a scaling factor of 0.5.

- 0.5x(n): Represents the current input scaled by 0.5.

- 0.25y(n − 1): Represents a feedback loop where the previous output is delayed and scaled by 0.25.

Check Given Equation is Variant or Invariant:

- Input → delay:

- delayed input → [System] → y1

- Input → System → y2

- Now make a delay of output 2 in k amount → output 3

- Check: Does output 1 = output 3?

- Example: y(n) = nx(n)

- y1(n,k) = nx(n − k)

- y2(n) = nx(n)

- y2(n − k) = (n − k) x(n − k)

- Since y1 ≠ y2, it is Variant (not invariant).

- Sampling, equation, encoder

- t = nT

- xa(t) = xa(nT) → sampling nth spike Time

- t = n/Fs​

- T→ কতক্ষণ পরপর একটা sample (How often a sample is taken)

- Fs = 1/T

- Here Fs → sampling rate or frequency

- For discrete time signal:

- Digital frequency f = F/Fs​

- So, 	ω = ΩT

- ω, Ω→ they are angular frequency

- Here F → analog frequency

- Frequency Ranges:

- For Continuous Time:

- − ∞ < F < ∞ Hz

- − ∞ < Ω < ∞

- and

- − Fs/2 < F < Fs/2​​

- − 1/2T < F < 1/2T​

- For Discrete Time:

- − 1/2 < f < 1/2​ Radian

- − π < ω < π

- and

- − π/T < Ω < π/T

- − πFs < Ω < πFs​

- Nyquist Sample Rate

- Fs = 2Fmax​(Fmax Means the Max Frequency between all the component)

- Ex 01: calculate Nyquist

- xa(t) = 3cos50πt + 10sin300πt − cos100πt

- (Calculation for each term):

- 3cos50πt ⟹  2πFt ⟹  F = 25

- 10sin300πt ⟹  F = 150

- −cos100πt ⟹  F = 50

- Fmax = 150

- Nyquist  = 2×150 = 300 Hz

- There have No sign effect Before sin and cos Function

- Folding Frequency = 2Fs​​

- Ex 02:

- xa(t) = sin480πt + 3sin720πt

- Fs = 600 Hz

- Nyquist rate, F = 2×Fmax = 720

Linear Time-Invariant (LTI) Systems

- Impulse Representation: Any sequence x(n)$can be represented as a sum of scaled and shifted unit impulses δ(n): x(n)=∑k=−∞∞​x(k)δ(n−k).

- Impulse Response h(n): The response of an LTI system to a unit impulse input δ(n).

- Convolution Sum: The output y(n) of an LTI system is the convolution of input x(n) and impulse response h(n).

- Formula: y(n) = x(n)∗h(n) = ∑k=−∞∞​x(k)h(n−k).

Sampling and Aliasing

- Sampling Theorem: To avoid aliasing, the sampling frequency must satisfy specific criteria.

- Aliasing Formula: Frequencies that are aliases of f_0 are given by f_0 + k f_s.

- Example: 10, 50, 90, 130 are all aliases if f_s = 40Hz.

- Normalized Frequency: f = F/F_s (Relative or normalized frequency).

Analysis of Discrete Time LTI system

- A Discrete-Time LTI system is a digital system that processes sampled signals in a linear and time-invariant way, and its analysis means studying how this system behaves and produces output for a given input using its impulse response and mathematical tools like convolution, difference equations.

- δ(n) impulse signal given to system → [unknown system] → h(n) get impulse response output

- Note: Once h(n) is known, that system is no longer unknown.)

- For any input x(n): x(n)→[h(n)] this will be y(n) = x(n) ∗h(n)

- Note: Output is found by convolving.)

Time-Domain Methods for Discrete Time Systems

- Convoluytion Sum

- Difference Equation

Convolution Sum

Used for LTI systems with known impulse response h[n].

Output: y[n] = ∑k=−∞∞ x[k]⋅h[n−k]

Principle: Sum contributions of all impulses in the input.

Difference Equations

Difference equation defines a system where output depends on past inputs and outputs, and its solution is the sum of complementary and particular parts.

Used when the system is defined recursively:

y[n] + a1y[n−1] + ⋯= b0x[n] + b1x[n−1] + …

Output computed step by step using past outputs and inputs.

General form:

y(n) + b1y(n−1) + b2y(n−2) + ⋯+ bNy(n−N) = a0x(n) + a1x(n−1) + ⋯+ aMx(n−M)

- M > N ⟹  M = order of system

- N > M ⟹  N = order of system

Solution Method:

y(n) = yc(n) + yp(n)

- yc(n) → complementary solve

- yp(n) → particular solve

Steps:

- i/p component (i.e. all the x(n) compat) = 0

- we assume y(n) = λn (λ→ lamda)

- Find the roots (also called igen value of the equation).

- Write down yc(n) in term of roots and unknown constants.

- solve to find the unknown constants.

Example: 2.4.9

y(n)−3y(n−1)−4y(n−2) = x(n)+2x(n−1) x(n) = 4nu(n)  N >M, Order of system = 2

Solution Steps:

- y(n)−3y(n−1)−4y(n−2) = 0

- let y(n) = λn

  - λn−3λn−1−4λn−2=0

  - λn−2(λ2−3λ−4)=0

  - λ2−3λ−4=0

- λ=4,−1

- yc​(n)=C1​(4)n+C2​(−1)n

- yp​(n) is a form of input

  - yp​(n)=Kn(4)nu(n)

  - K is distance of input and output.

Convolution: Convolution is the process of combining an input signal with a system’s impulse response to produce the output signal.

- y(n) = ∑k=−∞∞​x(k) h(n − k) = x(k) ∗h(k)

- y(n) = ∑k=−∞∞​h(k) x(n−k) = h(k)∗x(k)

- Both are same as it is LTI system, use the equation which is easy to fold or use.

Specialized Operations

- Circular Convolution: Used for periodic signals; can be solved using the Matrix Method.

- Deconvolution: Finding the unknown input sequence or impulse response given the output and the other known sequence.

- Properties of Convolution

Commutative: x(n) ∗h(n) = h(n) ∗x(n)

Distributive: x(n) ∗[h1(n) + h2(n)] = x(n) ∗h1(n) + x(n) ∗h2(n)

Associative: [x(n) ∗h1(n)] ∗h2(n) = x(n) ∗[h1(n) ∗h2(n)]

Shifting Property: If x1(n)∗x2(n) = z(n), then x1(n−k1)∗x2(n−k2)=z(n−k1−k2).

x[n]∗δ[n−k] = x[n−k]

Convolution with Impulse: x(n)∗δ(n−n1) = x(n−n1).

Scaling Property: δ(an) = |a| δ(n).

Circular Convolution

- x1(n)⊗x2(n) = X1(k)⋅X2(k) (DFT from x1 and x2)

- Must Follow N ≥ L for aliasing

- Note: To make circular convolution we nee to make length equal. If lengths are not equal, do zero padding

Example (Graphical/Circle Method):

- x1​(n)={2,2,1,2}

- x2​(n)={1,2,3,4}

- (Steps involve folding x2​ and shifting anticlockwise around a circle)

- y(0)=14

- y(1)=16

- y(2)=14

- y(3)=10

- Result: y(n)={14,16,14,10} (Arrow under 14)

Convolution Understanding                                       (This Section is not for exam)

Convolution is a process that tells us how a system responds to any input.

It combines:

- Input signal (x[n]) – the numbers you give the system

- System response (h[n]) – how the system reacts to a single input

Key idea:

Each input spreads over time according to the system. Whenever the effects of different inputs overlap, we add them to get the total output.

How the system works

A system is like a machine that reacts over time:

One input may create multiple outputs over several time steps

The “spread” of an input is determined by h[n]

Example:

System response: h[n] = [1, 2]

Input x[n] = 3 at step 0 → spreads over 2 steps: [3, 6]

Each new input also spreads → overlapping outputs are added

Example with multiple inputs

System: h[n] = [1, 2]

Input: x[n] = [3, 4, 2]

| Time | Contributions | Total Output y[n] |
| --- | --- | --- |
| 0 | 3 | 3 |
| 1 | 6 (from 3) + 4 (from 4) | 10 |
| 2 | 8 (from 4) + 2 (from 2) | 10 |
| 3 | 4 (from 2) | 4 |

Final output: y[n] = [3, 10, 10, 4]

Notice: each input spreads and overlapping contributions are added

Key points to remember

Each input spreads over time according to h[n]

Overlapping spreads are summed to produce output

The first output step only sees the first input

Later outputs may have contributions from multiple inputs

Convolution formula (y[n] = sum of x[k]*h[n-k]) is just a precise way to describe this “spread and sum” process

Intuition

Imagine the system is a machine with memory

Each input is a pebble thrown into a pond

Each pebble makes ripples that last multiple steps

The height of water at each step = sum of ripples from all pebbles

That’s convolution in simple terms

Why does spreading exist at all?

Because many real systems cannot react instantly and finish immediately.

They have duration.

Real-world intuition:

- Echo in sound → one clap spreads over time

- RC circuit → voltage doesn’t change instantly

- Heat transfer → effect spreads gradually

- Filters → smooth signals over time

So spreading = natural behavior of systems with memory

Key misunderstanding to fix

You said: “Systems never spread, they just do specific work”

Actually:

- Mathematically → every LTI system is defined by convolution

- Physically → spreading is how systems implement behavior over time

- So:

- Systems are NOT “randomly spreading”

- They are following their impulse response

In real systems, the impulse response h[n] is designed in such a way that, after convolution (multiplication and addition), the final output becomes the required system response.

So, we do not directly design the exact output. Instead, we design h[n], and convolution ensures the correct output for any input.

If we try to force an exact output directly, the system will not behave properly, because a real system must produce correct outputs for all possible inputs, not just one specific case.

Another Type Explain(Broad):

System response:

h[n]=[1,2]

Input:

x[n]=[3,4,2]

For Example:

I realize that at output time ( n = 2 ), I need to check the system’s response ( h[n] ) for all inputs that can contribute. So at ( n = 2 ), the system will produce responses from ( k = 0 ) and ( k = 1 ). I calculate the response for ( n = 2 ) and ( k = 0 ), then the response for ( n = 2 ) and ( k = 1 ), and finally I add them together. This is exactly how convolution works: each input spreads according to ( h[n] ), and the total output is the sum of all contributions.

Step-by-step table (n vs k)

| n | k | x[k] | h[n-k] | x[k]·h[n-k] | Contribution to y[n] |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 3 | h[0-0]=h[0]=1 | 3·1 = 3 | 3 |
| 0 | 1 | 4 | h[0-1]=h[-1]=0 | 4·0 = 0 | +0 |
| 0 | 2 | 2 | h[0-2]=h[-2]=0 | 2·0 = 0 | +0 |

Y[0] = 3

| n | k | x[k] | h[n-k] | x[k]·h[n-k] | Contribution to y[n] |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 3 | h[1-0]=h[1]=2 | 3·2 = 6 | 6 |
| 1 | 1 | 4 | h[1-1]=h[0]=1 | 4·1 = 4 | +4 |
| 1 | 2 | 2 | h[1-2]=h[-1]=0 | 2·0 = 0 | +0 |

Y[1] = 10

| n | k | x[k] | h[n-k] | x[k]·h[n-k] | Contribution to y[n] |
| --- | --- | --- | --- | --- | --- |
| 2 | 0 | 3 | h[2-0]=h[2]=0 | 3·0 = 0 | 0 |
| 2 | 1 | 4 | h[2-1]=h[1]=2 | 4·2 = 8 | +8 |
| 2 | 2 | 2 | h[2-2]=h[0]=1 | 2·1 = 2 | +2 |

Y[2] = 10

| n | k | x[k] | h[n-k] | x[k]·h[n-k] | Contribution to y[n] |
| --- | --- | --- | --- | --- | --- |
| 3 | 0 | 3 | h[3-0]=h[3]=0 | 3·0 = 0 | 0 |
| 3 | 1 | 4 | h[3-1]=h[2]=0 | 4·0 = 0 | 0 |
| 3 | 2 | 2 | h[3-2]=h[1]=2 | 2·2 = 4 | +4 |

Y[3] = 4

Final output y[n]=[3,10,10,4]

- Using n−k in the formula we automatically selects the correct element position of h[n] for each input.

- At each output time n, we measure the response of the system for each input k, multiply it with input, and add all contributions.

At output time n:

- k tells you which input you are currently looking at and k = 0 is first input and then so on others input in right. Here as k increase input will go to next from left to right.

- For nth time and for kth input the system response index position is h[n−k]. Here n = 0 is last input response or rigth side response. As k increase it go to left from right as input consumed one by one.

- Multiply x[k]⋅h[n−k] → that’s how much this input totally contributes to the output at time n.

- Then sum contributions from all relevant k → total output y[n].

| 3 (k = 0) | 4 (k = 1) | 2 (k = 2) |
| --- | --- | --- |

| 0 (k = 2) | 0 (k = 1) | 1 (k = 0) |
| --- | --- | --- |

Graphical Convolution: For each n = 0,1,2, calculate values and then make a figure.

Ex: 01

- h(n) = {1, 2, 1, −1} then if fold h(−k) = {−1, 1, 2, 1} (Note: Arrow is under 2)

- x(n) = {1,2,3,1}

- y(n) = ∑x(k) h(n−k) for various n

- Index of x(n) ⟹  0 to 3

- ndex of h(−n) = −2 to 1

- Index of Convolution = −2 to 3

- Here A variable is n

- And Another variable K But we must use k range from -inf to inf, and in this siyuation it is -2 to 3)

Tabular Method:

(A table is drawn with h(n) values 1, 2, 1, -1 across the top and x(n) values 1, 2, 3, 1 down the side)

| h(n)→ x(n)↓ | 1 | 2 | 1 | -1 |
| --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 1 | -1 |
| 2 | 2 | 4 | 2 | -2 |
| 3 | 3 | 6 | 3 | -3 |
| 1 | 1 | 2 | 1 | -1 |

- Result: y(n) = {1, 4, 8, 8, 3, −2, −1} (Note: Arrow is under 4)

Convolution with Unit Step (u(n)):

- u(n) Whatever is multiplied with u(n) it become a right sided signal

- Ex: h(n) = an u(n) ∣a∣<1,

- x(n) = u(n)

- y(n) = ∑k=−∞∞​x(k) h(n−k) —— ①

- y(n) = ∑k=−∞∞​x(n−k) h(k) —— ②

- Note: (Using formula ② is easier because folding/shifting x(n) is easy

  - y(0) = 1

  - y(1) = 1+a

  - y(2) = 1+a+a2

  - y(3) = 1+a+a2+a3

- Full Details: Show the first 4, then put the rest directly

- Correlation: Correlation measures how similar two signals are as one signal is 			  shifted over the other.

- Types:

- Cross-correlation: Similarity between two different signals (x and y).

- Auto-correlation: Similarity of a signal with itself.

Correlation:

- x(n)

- Reflected: y(n) = ax(n − D) + w(n)

- a→ attenuation

- D→ Delay

- w(n)→ is noise rxy(l) = start from Y

- rxy = ∑n=−∞∞ x(n)y (n−l)

      - l→ shift

Comparison to Convolution:

- Convolution:

- = ∑k=−∞∞​x(k)h (n−k)

- Step:

- h(k)→ h(−k) → h(−n + k)

- → Fold + shift

- Book Ref: 2.3.3, 2.6

Lec: 07: Time Domain to z-Domain

Z-Transform of Discrete-Time Signals

The Z-transform is a mathematical tool used to convert a discrete-time signal x[n] of time domain into a complex frequency domain representation.

X(z) = ∑n=−∞∞ x[n]z−n,	 z = rejθ (complex variable)

- Converts a time-domain sequence into a complex frequency-domain.

- Each exponent of z−n encodes the time index n.

- Each term z−n represents the time shift of the corresponding sample x[n].

- The magnitude and angle of z encode amplitude growth/decay and frequency content.

- Substituting z = ejω reduces the z-transform to the DTFT, showing the signal’s behavior at each frequency.

Properties of z-transform

- Linearity: a1x1(n) + a2x2(n) ⟷  a1X1(z) + a2X2(z)

- Convolution: x(n) = x1(n)∗x2(n) → Z(z) = X1(z)⋅X2(z) (Multiplication)

- Time Shifting: x(n−k) to Z transform will be z−kX(z)

- Scaling: anx(n) Z will be X(a−1z) (where a is the scaling factor).

- Time Reversal: x(−n) Z will be X(z−1)

- Differentiation in z-domain: nx(n) Z will be  −zdX(z)/dz​

- Rule: Differentiate X(z) and then multiply by -z.

- Example: X(z) = 1 − z−1 − z−6 + z−7 represents the sequence {1,−1,0,0,0,0,−1,1}.

Why we Use Z-Transform:

1. Converts Convolution into Multiplication

In time domain: y[n]=x[n]∗h[n]  → a sum over all k, which can be long or infinite.

In Z-domain:

Y(z) = X(z)⋅H(z)

Much easier to handle algebraically.

Especially useful for long sequences or recursive systems.

- Region of Convergence (ROC): The set of values in the z-plane for which the X(z) summation converges (is finite).

- Causal/Right-sided: ROC is the exterior of a circle (|z| > a). Causal signals include all values except z=0.

- Non-causal/Left-sided: ROC is the interior of a circle (|z| < a). Includes all values except z = infinity.

- Both-sided: Includes all values except z = 0 and z = infinity.

- Power Series Method: Also known as the long division method, used to convert X(z) back into x(n).

Example: X(z)=(1+2z−1)/(1−2z−1+z−2)​

2. Simplifies Difference Equations

Time-domain recursion can be tedious:

y[n]−0.5y[n−1] = x[n]

Z-transform converts it to an algebraic equation:

Y(z) − 0.5z − 1Y(z) = X(z)  Converted to Y(z) = X(z)1−0.5z−1

You can solve for Y(z) directly, then inverse Z-transform to get y[n].

3. Analyzes Stability and Causality

- Poles and zeros of H(z) tell you if the system is stable (poles inside unit circle)

- Shows if the system is causal or not

- But they are harder to see directly in time domain.

4. Frequency Response Analysis

This part is just explaining how frequency response comes from Z-transform and how all transforms are connected with it.

Step-by-Step Meaning

1. Start with Z-Transform

X(z) = ∑n=−∞∞ x[n]z−n

This is a general tool to analyze signals and systems.

- Put z = rejω

X(z)=∑x[n](rejω)−n

Now it looks like Fourier Transform, but with extra term r−n

3. Key Idea

X(z) = FT of x[n]r−n

Meaning:
Z-transform = Fourier Transform of a scaled signal

4. Special Case (Most Important)

If:		r = 1⇒z = ejωr

Then:		X(z) = ∑x[n]e−jωn

This becomes DTFT

Final Meaning of Your Line

“Substitute z = ejω

We are removing scaling (r) and only keeping frequency

Frequency response = Fourier Transform of impulse response

Simple Summary

- Z-transform: general (has r + ω)

- DTFT: special case (only ω)

- Frequency response: DTFT of h[n]

Continuous Time:

- F.T [x(t)] = ∫−∞∞​x(t)e−jωtdt

Discrete Time:

- F.T [x(n)] = ∑n=−∞∞​x(n)e−jωn

- Z.T [x(n)] = ∑n=−∞∞​x(n)z−n

- where, z = rejω

Example:

- Find z-transform and ROC of

- x(n) = anu(n) (a>0)

- X(z) = 11−az−1​ for ∣z∣>∣a∣

- = zz−a

- Zeros: z = 0 (represented by a circle symbol)

- Poles: z = a (represented by an X symbol)

Inverse z transform

The inverse z-transform is the process of recovering the time-domain signal x[n] from its z-transform X(z).

Using complex analysis, specifically the Cauchy integral theorem, the inverse z-transform can be expressed as:

- x(n) = 12πj∮X(z)zn−1dz → I.Z.T

Ex 5: X(z) = 2 − 1.5z−1 + 0.5z−2  → cannot solve as formula.

Using 1 and 3, X(z) can be brought into the table formula.

Pole Zero Cancellation:

H(z)=(z−1)(z+2)(z+2)(z+3)

- Ex: x(n) = (1/4)nu(n) + (−1/2)nu(n)

- =1−1/4z−11​+1+1/2z−11​ (solve separately as it is linear)

- =z−1/4z​+z+1/2z​=(z−1/4)(z+1/2)2z(z+1/8)​

- Zeros: z = 0, z = −1/8

- Poles: z = 1/4, z = −1/2

- (A plot shows the intersection of both common ROC areas)

- Ex: x(n)=(1/4)nu(n) − (−1/2)nu(−n−1):

- Type: Non causal, Both size signal.

- X(z) = 1−1/4z−11​−1−(−1/2)z−11​=z−1/4z​−z+1/2z​

- =(z−1/4)(z+1/2)2z(z+1/8)​ (same as previous ans)

- (A plot shows the ROC as a ring between the two poles)

Examples and ROC (Region of Convergence)

Ex 01: x(n) = {1,2,5,7,0,1} (Arrow under 1)

- X(z) = 1z−0+2z−1+5z−2+7z−3+0⋅z−4+z−5

- = 1+2z−1+5z−2+7z−3+z−5

- Ex 02: x(n) = {1,2,5,7,0,1} (Arrow under 7)

- X(z) = 1z−(−2)+2z−(−1)+5z−0+7z−1+0⋅z−2

- = z2+2z+5+7z−1

- Properties: converge → bounded → stable → finite.

Right, Left, and Both-Sided Signals

Ex: x(n) = (12)nu(n)

- Multiple with u(n) is a right sided signal.

- x(0) = 1, x(1) = 1/2, x(2) = 1/4…

- X(z) = 11−12z−1; ROC =∣z∣>1/2.

- Note: Table of ROC and formula exist in Book.

Causality and ROC:

- u(n) → causal signal.

- u(−n) → left sided signal → anti-causal.

- u(n) + u(−n) → Both sides → non-causal.

- Causal Case:

- If pole ∣z∣= r, ROC is all finite values where ∣z∣> r (Outside the circle).

- Anti-causal Case: ROC ∣z∣< r (Inside the circle).

- Non-causal Case: ROC will be a ring.

- Fourier Series: Used for periodic signals.

- Fourier Transform: Used for aperiodic signals.

- FT / DTFT: Transitions between the discrete time domain and continuous frequency domain.

- DFT: Sampling the continuous spectrum of DTFT/FT or X(ω) results in a discrete frequency representation.

- FFT: A fast algorithm used to compute the DFT.

Fundamental DTFT Formulas:

- x(n) Fourier Transform to X(ω)

- Analysis (DTFT): X(ω) = ∑n=−∞ ∞​x(n)e−jωn

- Synthesis (IDTFT):  x(n) = 1/2π ∫2π​X(ω)ejωn dω

- DFT Note: DFT goes from 0 to 2π with N number of samples.

Example for FT / DTFT

1. x(n) = αnu(n) (Right-sided signal):

- X(ω)=∑n=−∞∞​x(n)e−jωn

- =∑n=0∞​αnu(n)e−jωn

- =∑n=0∞​(αe−jω)n→ infinite series

- Result: X(ω) = 1/(1−αe−jω)

2. x(n) = δ(n) (Impulse signal):

- X(ω) = ∑n=−∞∞​δ(n)e−jωn

- = δ (0)e−jω(0) = 1⋅1=1

- Result: X(ω) = 1

- 3. x(n) = (1/2)nu(n) |

- Magnitude and Phase Spectrum:

- X(ω)=1−21​e−jω1​=1−21​(cosω−jsinω)1​=2−cosω+jsinω2​

- Magnitude: ∣X(ω)∣=(5−4cosω)1/22​

  - ∣X(ω)∣max​=(5−4)1/22​=2 (at ω=0)

  - ∣X(ω)∣min​=(5−(−4))1/22​=2/3 (at ω=π)

- Phase Spectrum: ∠X(ω)=tan−1(2−cosω−sinω​)

Advanced Problems

4. Find IDTFT for a Rectangular Frequency Pulse:

- Given X(ω)=1 for −ωc​≤ω≤ωc​ and 0 otherwise.

- x(n)=2π1​∫−ωc​ωc​​1⋅ejωndω

- =2π1​[jnejωn−e−jωn​]

- Result: x(n)=πnsinωc​n​ for n=0.

- For n=0: x(0)=πωc​​.

7. x(n) = u(n) (Unit Step):

- Analyzed as u(n)=ue​(n)+uo​(n) (Even and Odd parts).

- Uses properties of ue​(n)=21​[u(n)+u(−n)]=21​+21​δ(n).

- Transformation Flow:

- x(n) (Time domain, aperiodic, discrete)

- DTFT/FT​

- X(ω) (Freq domain, periodic, continuous)

- X(ω)

- sampling​

- X(k) (Freq domain, periodic, discrete) ← DFT

Fourier Series: Periodic (Convention)

Fundamental Concepts:

- 1. Continuous-Time Periodic Signal: Consists of an infinite number of frequency components, represented as x(t) = ∑k=−∞∞​Ckej2πkF0t

- Continuous-Time Fourier Series (CTFS)

- where:

- Ck=1/T∫0T x(t) e−j2πkF0t dt

- T = period

- F0 = 1/T = fundamental frequency

- Ck = Fourier coefficients

A periodic signal is made of many sine/cosine waves and Each term represents a frequency component.

F0 = fundamental frequency

kF0 = harmonics (multiples of base frequency)

- 2. Discrete-Time Signal Intervals: These are typically analyzed over the intervals (−π,π) or (0,2π).

So we only study one interval (called fundamental interval) and study of it is enough.

Key Equations:

- Synthesis Equation: Used to create a signal from its frequency components:

- x(n) = ∑k=0N−1 Ckej2πkn/N where N is the fundamental period.

- Analysis Equation: Used to decompose a signal into its frequency components

- Ck = 1/N∑k=0N−1 x(n)e−j2πkn/N.

Characteristics of Frequency Components (Ck)

Ck are the Fourier coefficients that represent how much of each frequency is present in a signal. Time-domain signal x[n] is converted into frequency components Ck​

- Properties:

- Ck is a complex variable possessing both magnitude and angle.

- Ck can be positive or negative to assist in mathematical derivations.

- Spectrum:

- The output spectrum Ck is discrete.

- Magnitude: Shows strength of each frequency and it is even symmetry.

- Phase: Shows phase (shift) of each frequency and it is odd symmetry.

- Mathematical Identity

- A key identity mentioned for these calculations is e−j2πn = 1.

- Fourier Series: Represents periodic signals as a sum of fundamental and harmonic frequencies.

- Fourier Transform (FT): The Fourier Transform is a mathematical operation that converts a continuous-time aperiodic signal into its continuous frequency-domain representation.

- Pair Formula:

- X(ω) = ∫−∞∞  x(t)e−jωtdt

- Meaning:

- x(t) → time domain

- X(ω) → frequency domain

- Shows what frequencies exist in the signal

- FT: Continuous aperiodic → continuous aperiodic spectrum

- DTFT: Discrete aperiodic → continuous periodic spectrum

- DFT: Finite discrete → discrete periodic spectrum

- Relationship to Z-Transform: The Z-transform is equivalent to the Fourier Transform when r = 1 (i.e., evaluated on the unit circle).

- Forward Fourier Transform

- X(ω)=∫−∞∞  x(t) e−jωt dt

- Inverse Fourier Transform

- x(t)=12π∫−∞∞X(ω) ejωt dω

DTFT

Discrete-Time Fourier Transform (DTFT) is used for the analysis of finite energy, aperiodic signals.

This transform transitions signals from the Time Domain (x(n)) to the Frequency Domain (X(ω)), converting discrete, aperiodic data into a continuous, periodic frequency spectrum.

1. Fundamental Equations

The notes provide two primary equations for moving between domains:

- Analysis Equation (DTFT): Converts time-domain samples into frequency components.

- X(ω) = ∑n=−∞ ∞​x(n)e−jωn

- Synthesis Equation (Inverse DTFT): Reconstructs the time-domain signal from the frequency domain.

- x(n) = 1/2π ∫2π​X(ω)ejωn dω

X(ω) is decomposition of X(t) into its frequency component.

2. Characteristics and Properties of X(ω)

- Complex Variable: X(ω) is a complex variable with both magnitude (even symmetry) and phase (odd symmetry).

- Continuous Spectrum: Unlike the discrete output of a Fourier Series, the DTFT spectrum is continuous.

- Periodicity: The spectrum X(ω) is periodic with a period of 2π, meaning X(ω)=X(ω+2πk).

- Frequency Range: The unique frequency range is typically analyzed over the interval (−π,π).

3. Comparison: CTFT vs. DTFT

The notes distinguish between the Fourier Transform for Continuous-Time (CT) and Discrete-Time (DT) signals:

- CTFT: Uses an integral of terms and has a frequency range from −∞ to ∞.

- DTFT: Uses a summation of terms for analysis and is unique over the interval (−π,π).

5. Practical Example

The signal x(n)={−1,2,5,2,−1} (with the arrow under 5):

- Expansion: X(ω) = −1e j2ω + 2ejω + 5 + 2e−jω − 1e−j2ω.

- Simplification: Using Euler's identities, this simplifies to:

- X(ω) = 5 + 4cos(ω) − 2cos(2ω).

- Calculated values using table for plotting:

- At ω=0, X(ω)=7.

- At ω=π/2, X(ω)=7.

- At ω=π, X(ω)=−1.

Discrete Fourier Transform (DFT)

Definition:

The Discrete Fourier Transform (DFT) converts a finite discrete-time signal into its discrete frequency components.

Idea: Any finite discrete signal can be written as a sum of complex exponentials (sinusoids) with different frequencies.

- Continuous vs Discrete Frequency Sampling

- A signal can have a continuous frequency spectrum, but in practice we often sample it at discrete points to get a finite representation.

- Continuous and periodic spectrum Comes from DTFT / Fourier Transform : X(ω)

- Discrete Frequency Spectrum (DFT): X(k)

- Number of samples: N

- Each frequency step or frequency resolution: Δω = 2π/N

- Ωk = 2πk/N, k = 0, 1, 2,…, N−1

- X(2πk/N) = X(ω),  k = 0,1,…,N−1

- So by sampling X(ω) at N points, we get the discrete version X(k).

- 2. DFT Formula

- For a discrete-time sequence x(n) of length N:

- X(k) = n=0N−1x(n)e−j2πkn/N, k = 0, 1 ,…, N−1

- WN = e−j2π/N → Twiddle factor

- Using WN:

- X(k) = n=0N−1x(n) W*knN

- The DFT converts time-domain samples x(n) into frequency-domain samples X(k).

- 3. Inverse DFT (IDFT)

- X(k) = 1Nn=0N−1x(n) W*knN

x(n) → time-domain sequence

X(k) → frequency-domain components

N → number of samples

n → time index

m → frequency index

- W*N = e−j2π/N → complex conjugate of the twiddle factor

Magnitude

∣X(m)∣= (Real)2+(Imag)2​

Shows strength of each frequency

Phase

θ(m) = tan−1( ImaginaryReal)

Shows phase (angle/shift) of each frequency

Ex: Compute the DFT

- x(n) = {0,1,2,3}

- N=4

Twiddle Factor Calculation (WN = e−j2π/N):

- For N = 4:

- W40​= e−j42π​(0)=cos0−jsin0=1

- W41​= cos2π​−jsin2π​=−j

- W42​= cosπ−jsinπ=−1

- W43​= cos23π​−jsin23π​=j

- W4​= 1111

- 1−j−1j​

- 1−11−1​

- 1j−1−j​​

- Y(k) = WN.X(n)T

- = 6

- −2+2j

- −2

- −2−2j → Discrete get by sampling

DFT Leakage: Occurs when signal frequencies do not align with DFT bins, causing energy to spread to adjacent bins.

Windowing: Techniques used to reduce DFT leakage.

Types: Rectangular, Hamming, Triangular, etc.

DFT Resolution: Can be increased using Zero Stuffing.

DFT Realization and Understanding           (This Section is not for exam)

- Discrete Fourier Transform (DFT) is a tool used to analyze discrete-time signals in the frequency domain.

- It tells us how much of each frequency component is present in a signal.

- The DFT is essentially a correlation operation between the signal and a set of orthogonal complex sinusoids

DFT as Correlation (Inner Product)

The summation in DFT:

- X(k) = n=0N−1x(n)e−j2πkn/N, k = 0, 1 ,…, N−1

is essentially a correlation between:

The original signal is: x(n)

The complex sinusoid signal component is: e−j2πkn/N

Interpretation:

High correlation → signal contains that frequency → |X(k)| is large

Low correlation → signal does not contain that frequency |X(k)| ≈0

Understanding the Complex Exponential

Euler’s formula:

ejθ = cos(θ) + jsin(θ)

- So, e−j2πkn/N is a complex sinusoid

- Contains both cosine and sine → can represent any signal with phase

- Frequency = k/N → completes k cycles in N samples

Matching Frequency

If signal contains frequency m/N:

x(n) = Aej2πmn/N

Compute DFT:

- X(k) = n=0N−1Aej2πmn/Ne−j2πkn/N = An=0N−1ej2π(m−k)/N

If  k = m:

X(m) =n=0N−1A = AN

All terms add constructively → large magnitude → strong frequency

Case 2: Non-Matching Frequency

If  k≠m:

X(k) = An=0N−1ej2π(m−k)/N

- Exponential rotates in the complex plane

- Positive and negative terms cancel → |X(k)| ≈0

- Frequency is weak or absent

- In DFT we compute X(k) for predefined k = 0,...,N−1 Each k corresponds to a frequency k/N. The value X(k) measures how much of that frequency exists in the signal.

Frequency Resolution and Spectral Leakage in DFT

1. Frequencies measured by DFT

For a signal with N samples, the DFT computes X(k)

Each (k) corresponds to a discrete frequency
	fk = k/N
These are called DFT frequency bins.

A bin is simply: A specific frequency slot where the DFT measures how much of that frequency exists in the signal.

DFT divides the frequency range into N small sections, Each section = one bin

2. What DFT actually measures

- The DFT checks N predefined sinusoidal frequencies and measures how strongly each one exists in the signal.

- Large (X(k)) → that frequency exists strongly

- Small (X(k)) → weak component

- (X(k)=0) → that frequency does not exist

3. When the signal frequency matches a DFT bin

If the signal frequency is exactly
	f = k/N

then the energy appears only in one bin (X(k)).

This gives a clean spectrum.

4. When the frequency is not exactly (k/N)

If the signal frequency lies between two bins, the energy spreads across multiple bins.

This phenomenon is called Spectral Leakage.

Example:

True frequency = (2.5/N)

| k | value |
| --- | --- |
| 1 | small |
| 2 | medium |
| 3 | medium |
| 4 | small |

Energy appears in bins:

(X(2))

(X(3))

nearby bins instead of a single peak.

5. Important idea

The frequency is not lost, but the DFT cannot represent it exactly, so its energy leaks into neighboring frequencies.

6. How to reduce leakage

Increase N (more samples → better frequency resolution)

Apply window functions (Hamming, Hanning, Blackman)

Use zero padding for smoother spectra

Fast Fourier Transform (FFT)

- Efficiency: DFT calculation complexity is O(N^2), whereas FFT reduces it to N/2 log_2 N.

- Radix-2 FFT: A common FFT algorithm that uses a Butterfly Diagram to simplify computations.

- Twiddle Factor: WN = e−j2π/N

FFT (Fast Fourier Transform):

- Definition: Algorithm to compute DFT when N is very large.

- Concept: FFT decomposes the complex problem into smaller DFTs.

- Decomposition Tree: N-point → N/2-point → N/4-point.

- Requirement: FFT is applied if N is an integer multiple of 2 (N = 2k).

- Method: We use butterfly diagrams to compute FFT.

- Radix(Base): Radix can be 2, 4, 6, or 8.

Ex: 8-Sample Radix-2 Decimation

If x(n) has 8 samples (8 = 23):

Level 1: Two 4-point DFTs.

Level 2: Four 2-point DFTs.

Sample Separation (Bit Reversal/Decimation):

x(n) = {x(0), x(1), x(2), x(3), x(4), x(5), x(6), x(7)}

Even Samples (x1): {x(0), x(2), x(4), x(6)}

Odd Samples (x2): {x(1), x(3), x(5), x(7)}

Further Decomposition:

f1​(n)={x(0), x(4)}

f2​(n)={x(2), x(6)}

f3​(n)={x(1), x(5)}

f4​(n)={x(3), x(7)}

N-point DFT Formula: XN[k] = ∑n=0N−1​x[n]WNkn​ where WN = e−j2π/N​

FFT Recursive Formulas

The DFT sum is split into even and odd parts:

XN[k] = ∑h=0N/2−1​x[2h]WN2kh + ∑h=0N/2−1​x[2h+1]WN(2h+1)k​

Key Equations for k = 0, 1, …, 2N − 1:

- XN[k] = G[k] + WNk H[k]

- XN[k+2N] = G[k] − WNk H[k]

Butterfly Diagram Example

Problem: x(n) = {1, 2, 3, 4, 4, 3, 2, 1}, N = 8 = 23.

- Input Order (Bit-Reversed):

- x(0) = 1, x(4) = 4, x(2) = 3, x(6) = 2, x(1) = 2, x(5) = 3, x(3) = 4, x(7) = 1.

- Stage 1: Pairs are Convolved (e.g., 1+4=5 and 1−4=−3).

- Stage 2: Uses W40​ and W41​.

- Stage 3: Uses W80​,W81​,W82​,W83​.

- Result: X(k)={20,…}

Filter Types

- Finite Impulse Response (FIR):

- Depends only on present and past inputs (no feedback).

- Always stable.

- Impulse response has finite length.

- Infinite Impulse Response (IIR):

- Depends on present/past inputs and past outputs (feedback).

- More efficient but can be unstable.

- Recursive structure.

Section 6.3: Filter Realization Structures

- Direct Form I: Implements the numerator and denominator sections separately with separate delay lines 1.

- Direct Form II: An improved, memory-efficient structure that combines delay elements into a single common delay line 1.

Chapter 7: Adaptive Filters

Section 7.1: Adaptive Filter Concepts

- Core Definition: A digital filter (FIR or IIR) whose weights/coefficients are automatically adjusted by an Adaptive Algorithm 1.

- Mechanism: It minimizes the Mean Square Error (MSE) between a desired signal $d(n)$ and the actual output $y(n)$ 1.

- Performance Criteria:

- Stability 1.

- Quality 1.

- Speed of Adaptation 1.

- Tracking Capability 1.

Section 7.2: LMS Algorithm (Steepest Descent Method)

- Steepest Descent: An optimization technique used to find the minimum value of a cost function (MSE) 1.

- Update Rule: $w_{n+1} = w_n + \mu E\{e(n) x^*(n)\}$, where $\mu$ is the step size 1.

Section 7.3: Applications of Adaptive Filters

- Noise Cancellation: Removing unwanted noise from a signal (e.g., $50$ Hz noise from a $5$ Hz signal) 1.

- System Identification: Modeling an unknown system by adjusting filter weights to match its behavior 1.

- Channel Equalization: Removing distortion caused by communication channels 1.

Lecture: 14 Filter

1. Finite Impulse Response (FIR) Filters

Definition: An FIR filter is a digital filter whose impulse response h(n) has a finite duration, meaning it is zero outside of a specific finite time interval. If the input to an FIR filter becomes all zeros, the output will eventually become zero as well.

Core Concept:

- Memory: FIR filters have a "finite memory," typically viewing only the most recent M input samples to calculate an output.

- Realization: They are generally implemented using non-recursive structures (also called transversal or tapped-delay line filters).

- Key Advantages: They are guaranteed to be stable and can be designed to have exactly linear phase, meaning they do not distort the timing of different frequency components.

Formula:

- Difference Equation: The output y(n) is a weighted sum of current and past input samples: y(n) = k=0M-1 bk x(n-k)

- Transfer Function: In the z-domain, it is characterized by an all-zero system: H(z) = k=0M-1 bk z^{-k}

Example: A Moving Average Filter is a classic FIR filter. For a 4-point averager, the formula is: $$y(n) = \frac{1}{4}[x(n) + x(n-1) + x(n-2) + x(n-3)]$$ This filter behaves as a simple lowpass filter by smoothing out sudden changes in the input signal.

2. Infinite Impulse Response (IIR) Filters

Definition: An IIR filter is a digital filter where the impulse response h(n) is of infinite duration. Theoretically, even if the input becomes zero, the output can remain nonzero forever due to internal feedback.

Core Concept:

- Feedback: Unlike FIR filters, IIR filters use recursive structures where the current output depends on current/past inputs AND previous output samples.

- Efficiency: IIR filters are highly efficient; they can achieve much sharper frequency roll-offs with fewer multiplications and less memory compared to FIR filters.

- Key Risks: They are not guaranteed to be stable (poles must be inside the unit circle) and they generally exhibit nonlinear phase response.

Formula:

- Difference Equation: y(n) = k=0M-1 bk x(n-k) + k=1N ak y(n-k)

- Transfer Function: In the z-domain, it is a pole-zero system: 	H(z)=1−∑k=1N​ak​z−k∑k=0M​bk​z−k​

- (Note: The denominator coefficients $a_k$ determine the system's poles and stability.)

- Example: A simple Accumulator (or Digital Integrator) is an IIR filter: y(n) = x(n) + y(n−1) Another example is a 1st-order lowpass filter, such as y(n) = 0.22x(n) + 0.25x(n-1) + 0.87y(n-1), which uses feedback to sustain its response.

Summary Comparison for Exams

| Feature | FIR Filter | IIR Filter |
| --- | --- | --- |
| Impulse Response | Finite duration | Infinite duration |
| Structure | Non-recursive (no feedback) | Recursive (uses feedback) |
| Stability | Always stable | Must be designed for stability |
| Phase Response | Can be exactly linear | Usually nonlinear |
| Efficiency | Lower (needs more taps) | Higher (sharper with fewer taps) |
| Z-plane | Zeros only | Both Poles and Zeros |

1. Ideal Low-Pass Filter and Derivation

The ideal frequency response of a low-pass filter is:

Hd(f) = {1∣f∣≤ fc

0∣f∣> fc

- Fc  =  cutoff frequency (in Hz or normalized frequency)

- Hd(f) = desired frequency response

To get the impulse response, take the inverse Fourier transform of the ideal frequency response:

hd[n] = sin(2πfc(n − α))π(n − α)

Where:

n = sample index

a = M2 (M = filter order)

This formula is known as the sinc function, which is the ideal impulse response of an LPF.

Since the ideal response is infinite in length, we use a window function (like Hamming or Hanning) to truncate it and make it finite:

h[n] = hd[n]⋅w[n]

Where w[n] is the window function.

2. FIR LPF Formula

For a practical FIR low-pass filter of order M:

y[n] = k=0Mh[k] ⋅x[n−k]

Where h[k] is computed using:

h[k] = sin(2πfc(k − M/2))π(k − M/2) ⋅w[k],  k = 0. 1. 2 ….. M

- x[n−k] = input sample delayed by k

- y[n] = filtered output

- w[k] = window to limit length

3. Example

Design a 5-tap low-pass FIR filter:

Filter order M=4M = 4M=4 (so 5 taps)

Normalized cutoff frequency fc = 0.25

Step 1: Compute ideal impulse response (hd[n]):

hd[n] = sin(2πfc(n − 2))π(n − 2),    n = 0, 1, 2, 3, 4

Calculations:

hd[0] = 0

hd[1] = 0.159

hd[2] = 0.25 (using limit)

hd[3] = 0.159

hd[4] = 0

So, impulse response h[k] = [0, 0.159, 0.25, 0.159, 0]

Step 2: Apply to input sequence

Input: x[n]=[1,2,3,4,5]

Calculate Output Using Formula:

y[n] = k=04h[k] ⋅x[n−k]

Compute first few values:

y[0] = 0⋅1+0⋅0+...=0

y[1] = 0.159∗1+0.25∗0+...=0.159y

y[2] = 0.159∗2+0.25∗1+...=0.568y

And so on. You can continue for all samples.

The result is a smoothed version of the input, with high-frequency jumps reduced.

High Pass FIR Filter

A high-pass FIR filter is a finite impulse response filter whose coefficients are chosen so that low frequencies are suppressed and high frequencies pass.

- It is non-recursive (depends only on current and past input samples)

- Always stable and can have linear phase

Mathematically:

Definition: An FIR filter whose output is a weighted sum of current and past input samples, with coefficients h[k]h[k]h[k] designed to approximate an ideal high-pass frequency response.

1. Ideal Low-Pass Filter and Derivation

The ideal frequency response of a low-pass filter is:

Hd(f) = {0∣f∣≤ fc

1∣f∣> fc

- Fc  =  cutoff frequency (in Hz or normalized frequency)

- Hd(f) = desired frequency response

To get the impulse response, take the inverse Fourier transform of the ideal frequency response:

- Hd(f) is the ideal frequency response.

- But to implement the filter in time domain, we need weights for the input samples.

- That’s where hd[n]comes in.

hd[n] is the time-domain impulse response derived from ideal frequency response Hd(f) using the inverse Fourier transform.

Mathematically:

hd[n] = −1/21/2Hd(f)ej2πfndf

After solving this integral, you get:

hd[n] = sin(π(n−α))−sin(2πfc(n−α)) π(n−α)

Where:

n = sample index

a = M2 (M = filter order)

The first term sin(π(n−α))/π(n−α) ensures high-pass behavior by inverting low 	frequencies.

Again, the ideal response is infinite, so we multiply by a window function w[n] to truncate it:

h[n] = hd[n]⋅w[n]

Where w[n] is the window function.

Now, all things are ssme for High Pass Filter as it is same as low pulse filter rather difference in hd[k] formula.

A band-pass FIR filter is a finite impulse response filter whose coefficients are chosen to pass a specific frequency band and block everything else.

FIR formula is the same:

h[n] = hd[n]⋅w[n]

Only h[k] (the impulse response) is designed for band-pass behavior.

3. Derivation / Impulse Response

The ideal band-pass frequency response is:

Hd(f) = {1  f1 ≤ fc ≤ f2

0  otherswise

Where:

f1 = lower cutoff frequency

f2 = upper cutoff frequency

To get the time-domain impulse response hd[n], take the inverse Fourier transform:

hd[n] = sin(2πf2(n−α))−sin(2πf1(n−α)) π(n−α)

  - α=M/2 (center of filter)

  - The difference of sinc functions creates a band-pass effect in time domain.

  - This is sometimes called the “sinc-difference” method.

Just like in low-pass and high-pass filters, hd[n] time-domain coefficients derived from frequency specifications.

We usually multiply by a window function w[n] to limit the length:

h[n]=hd[n]⋅w[n]

4. FIR BPF Formula

Once h[n] is computed, the FIR formula is:

y[n] = k=0Mh[k] ⋅x[n−k]

Where:

  - x[n−k] = input sample delayed by kkk

  - h[k] = band-pass filter coefficient

  - y[n] = filtered output

5. Example

Design a simple 5-tap band-pass FIR filter:

Order M=4M = 4M=4 → 5 coefficients

Lower cutoff f1=0.2f_1 = 0.2f1​=0.2

Upper cutoff f2=0.4f_2 = 0.4f2​=0.4

Step 1: Compute hd[n] for value of n = 0,1,2,3,4

Approximate values (rounded):

hd[0]=0h_d[0] = 0hd​[0]=0

hd[1]=0.1h_d[1] = 0.1hd​[1]=0.1

hd[2]=0.3h_d[2] = 0.3hd​[2]=0.3

hd[3]=0.1h_d[3] = 0.1hd​[3]=0.1

hd[4]=0h_d[4] = 0hd​[4]=0

These are weights to combine past and current input samples.

Step 2: Apply FIR formula

Input: x[n]=[1,2,3,4,5]

Output:

y[n]=∑k=04h[k]⋅x[n−k]

y[0]=0⋅1+...=0

y[1]=0.1∗1+0.3∗0+… =0.1

y[2]=0.1∗2+0.3∗1+...=0.5

…and so on.

The output keeps the frequencies between f1and f2while suppressing lower and higher frequencies.

IIR Low-Pass Filter

1. Concept

An IIR (Infinite Impulse Response) filter is a digital filter where the output depends on both past inputs and past outputs.

Because the filter uses feedback (previous outputs), its impulse response theoretically continues forever, which is why it is called infinite impulse response.

For a low-pass IIR filter:

Low frequencies pass through

High frequencies are attenuated

Typical uses:

Noise reduction in audio

Signal smoothing

Sensor data filtering

2. Definition

A low-pass IIR filter is a recursive digital filter that passes low-frequency components while suppressing high-frequency components, and its output depends on both previous input samples and previous output samples.

3. General IIR Filter Formula

The general difference equation of an IIR filter is

y[n] = b0 x[n] + b1 x[n−1] + b2 x[n−2] + ... + bM x[n−M]
− a1 y[n−1] − a2 y[n−2] − ... − aN y[n−N]

Where:

- x[n] = input signal

- y[n] = output signal

- b coefficients = feedforward coefficients

- a coefficients = feedback coefficients

Key point:

FIR uses only input samples, but IIR uses input + past outputs.

4. Simple First-Order Low-Pass IIR Filter

A very common simple IIR low-pass filter is:

y[n] = α x[n] + (1 − α) y[n−1]

Where:

- 0 < α < 1

- α controls the cutoff frequency

Meaning:

- Current input contributes α

- Previous output contributes (1 − α)

So the filter smooths the signal over time.

5. Transfer Function (Z-Domain)

Taking the Z-transform:

Y(z) = α X(z) + (1 − α) z⁻¹ Y(z)

Rearranging:

Y(z)[1 − (1 − α)z⁻¹] = αX(z)

So the system function is

H(z) = Y(z) / X(z)

H(z) = α / (1 − (1 − α)z⁻¹)

This shows the filter contains a pole, which causes the impulse response to be infinite.

6. Impulse Response

If the input is an impulse:

x[n] = δ[n]

The output becomes:

h[n] = α(1 − α)ⁿ

for n ≥ 0

Since this sequence never becomes exactly zero, the impulse response is infinite.

That’s why it is called IIR.

7. Example

Let α = 0.3

Filter equation:

y[n] = 0.3x[n] + 0.7y[n−1]

Input signal:

x[n] = [1, 2, 3, 4]

Initial condition:

y[−1] = 0

Step-by-step calculation:

y[0] = 0.3(1) + 0.7(0) = 0.3

y[1] = 0.3(2) + 0.7(0.3)
= 0.6 + 0.21
= 0.81

y[2] = 0.3(3) + 0.7(0.81)
= 0.9 + 0.567
= 1.467

y[3] = 0.3(4) + 0.7(1.467)
= 1.2 + 1.027
≈ 2.227

Output sequence:

y[n] ≈ [0.3, 0.81, 1.467, 2.227]

Notice how the output becomes smooth, which is the behavior of a low-pass filter.

8. Key Characteristics of IIR Filters

Advantages

- Requires fewer coefficients

- Efficient computation

- Can approximate analog filters (Butterworth, Chebyshev)

Disadvantages

- May become unstable

- Phase response usually non-linear

Window Types and Formulas

The notes list three primary window functions used to truncate the infinite impulse response:

- Rectangular Window (wR(n)):

- wR(n) = 1 for 0 ≤ n ≤ M−1 (where M is the filter order).

- wR(n) = 0 otherwise.

- Hamming Window (wH(n)):

- wH(n) = 0.54−0.46cos[M−12πn] for 0 ≤ n ≤ M−1.

- Hanning Window (whan(n)):

- whan(n) = 0.5−0.5cos[M−12πn] for 0 ≤ n ≤ M−1.

Worked Example: Low Pass Filter (LPF)

Given: H(ω)=e−j2ω for the range −π/4≤ω≤π/4.

1. Calculate Ideal Impulse Response (h(n)):

- h(n)=2π1​∫−π/4π/4​e−j2ωejωndω=2π1​∫−π/4π/4​ejω(n−2)dω

- After integration and using the sine identity (sinθ=2jejθ−e−jθ​):

  - h(n)=π(n−2)1​sin[4π​(n−2)]

2. Applying the Window:

- If H(ω)=e−jαω, then the delay α=2M−1​. For this example, α=2, so M=5.

- The actual filter coefficients are found by: h′(n)=wR​(n)×h(n) for n=0 to 4.

- Handling n=2 (The Center Point): The formula becomes undefined (sin0/0). In this case:

  - h(2)=2π1​∫−π/4π/4​1⋅dω=2π1​[4π​−(−4π​)]=2π1​⋅2π​=1/4=0.25.

3. Symmetry Note: The notes emphasize that in the calculation table, the values of h(n) will be symmetric around the center point.

Lec: 15 | Design FIR → Hamming window

Problem Specification:

- Ideal Frequency Response: H(ω)=e−j3ω for the frequency range 43π​≤∣ω∣≤π.

- This represents a high-pass filter because it allows frequencies between 3π/4 and π.

1. Calculate the Ideal Impulse Response (h(n)): The impulse response is found using the Inverse Discrete Fourier Transform (IDFT): h(n)=2π1​∫−ππ​H(ω)ejωndω Because the filter is defined in two segments (negative and positive frequency), the integral is split: h(n)=2π1​[∫−π−3π/4​H(ω)ejωndω+∫3π/4π​H(ω)ejωndω] Resulting Formula: h(n)=π(n−3)1​[sinπ(n−3)−sin43π​(n−3)],

2. Determine Filter Order (M):

- Given the phase delay α=3.

- Using the relationship α=2M−1​, we find:

  - 3=2M−1​⟹6=M−1⟹M=7.

- The filter coefficients will be calculated for n=0 to 6.

3. Apply the Hamming Window (wham​(n)): The formula for the Hamming window is: wham​(n)=0.54−0.46cos(M−12πn​) For M=7, this becomes: wham​(n)=0.54−0.46cos(62πn​)

4. Final Filter Coefficients (h′(n)): The practical filter coefficients are obtained by multiplying the ideal response by the window: h′(n)=h(n)×wham​(n)

- Symmetry: The notes indicate that the results will be a mirror image around the center point (n=3).

- Center Point (n=3): At this value, the h(n) formula becomes undefined (0/0). It must be solved by taking the limit or direct integration at that specific point.

1. What LTI System Analysis Means

We’re analyzing linear, time-invariant systems, which are special because:

- Linearity: The system obeys superposition. If you know the outputs for certain inputs, you can combine them to find the output for a sum of inputs.

- Time-invariance: The system behaves the same over time. Delaying the input delays the output by the same amount.

Because of these two properties, the response of an LTI system can be fully described by how it reacts to a unit sample signal (an impulse).

2. Two Methods to Analyze LTI Systems

Method 1: Solve the difference equation

The system is defined by an equation like:

y(n)=F[y(n−1),y(n−2),...,x(n),x(n−1),...]

- Here F is a function of past outputs and present/past inputs.

- If the system is LTI, it can often be written as:

i=0∑N​ai​y(n−i)=j=0∑M​bj​x(n−j)

This is called a difference equation. Solving it gives the output for a given input.

Method 2: Decompose input into elementary signals

- Break the input x(n) into simple components (often impulses or exponentials):

x(n)=k∑​ck​xk​(n)

- ck​ are weighting coefficients.

- xk​(n) are elementary signals (like impulses).

- Find the output for each elementary signal:

yk​(n)=T[xk​(n)]

- Use linearity to combine the outputs:

y(n)=k∑​ck​yk​(n)

This is called the convolution sum method.

Why Use Unit Sample (Impulse) Response?

Any discrete long sequence of signal can be written as a sum of shifted and scaled impulses. So, if we know the system’s response to a unit impulse, we can find the response to all input. This is the foundation of convolution.

Step-by-Step Computation of Convolution

To compute y(n0​) (output at a specific time n0​):

- Folding: Flip the impulse response: h(k)→h(−k).

- Shifting: Shift the folded response by n0​: h(−k)→h(n0​−k).

- Multiplication: Multiply element-wise with the input: x(k)⋅h(n0​−k).

- Summation: Sum over all k to get the output:

y(n0​)=k=−∞∑∞​x(k)h(n0​−k)

- Repeat this for each time n to get the full output sequence.

5. Important Notes

- Convolution is commutative, i.e.,

- x(n)∗h(n)=h(n)∗x(n)

- You can choose to fold either x(n) or h(n) — the result will be the same.

Note: Fold h, then keep x values at up position and under it keep h values, and slide h from left to right. At first step the last value of h is under the first value of x. Now multiply h and x and sum them up. Total sum is the the step value. In this sequence calculate by sliding one by one.

Summary:

The convolution sum is the standard method to find the output of any LTI system:

y(n)=k=−∞∑∞​x(k)h(n−k)​

- Folding + shifting + multiplying + summing are the steps.

- Works for finite or infinite sequences.

- Fundamental in digital signal processing for LTI systems.

5. Solving a Simple Recursive System

Take:

y(n)=ay(n−1)+x(n)

Assume:

- Input applied for n≥0

- Initial condition exists: y(−1)

Now compute step by step:

y(0)=ay(−1)+x(0) y(1)=ay(0)+x(1)

Substitute y(0):

y(1)=a2y(−1)+ax(0)+x(1) y(2)=a3y(−1)+a2x(0)+ax(1)+x(2)

Continue pattern:

y(n)=an+1y(−1)+k=0∑n​akx(n−k)

This has two parts.

why, in a signal processing system, do we sometimes need to “remember” or use past values? Why can’t we just look at the current input? Let’s go step by step.

1. Signals often depend on the past

Many real-world signals don’t just depend on what’s happening now, they depend on what happened before.

Examples:

- Temperature sensor: the current reading is influenced by previous temperatures if you are smoothing or averaging.

- Audio echo effect: the sound we hear now depends on past sounds.

- Digital filters: to remove noise or enhance certain frequencies, the system must combine current and past inputs.

So, systems often need memory of past inputs or outputs to behave correctly.

2. The role of past values in systems

- Past inputs help the system know the history of the signal.

  - Example: cumulative average y(n)=n+11​∑k=0n​x(k)

    - To calculate the average at time n, you need all previous samples.

- Past outputs help the system maintain feedback or recursion.

  - Example: y(n)=ay(n−1)+x(n)

    - Using y(n-1) compresses all the past input information into one value.

    - This is much more memory-efficient than storing all previous inputs.

3. Why we can’t ignore past values

- Some operations are inherently cumulative or recursive.

  - Smoothing, averaging, filtering, echoes, IIR systems.

- If we ignore past values, the system cannot perform these operations correctly.

- In terms of equations:

  - FIR system → output depends on a finite number of past inputs

  - IIR system → output depends on past inputs AND past outputs

So past values are essential for capturing the signal’s history.

4. How difference equations help

- Instead of storing all past inputs, a difference equation uses previous outputs (y(n-1), y(n-2), …).

- The previous outputs encode all the necessary history.

- This is memory-efficient and practical for digital systems.

Why difference equations?

- They describe how the output changes from one sample to the next.

- They can handle feedback (recursive systems) efficiently.

- They allow us to compute the output step by step, only storing necessary past values.

Example:

y(n)=ay(n−1)+x(n)

- Using difference equations, we can compute y(0), y(1), y(2), … without storing all previous x(n).

- Trying to solve this using a differential equation is not practical because the system is discrete.

How past values carry history

Take a simple system:

y(n)=ay(n−1)+x(n)

- Here, y(n−1) is the previous output, not the raw past input.

- But y(n−1) already contains all the effect of previous inputs, because:

why do the powers of a grow in

y(n)=x(n)+ax(n−1)+a2x(n−2)+a3x(n−3)+…

Why powers of a appear

- Every time we go one step back, the previous output is multiplied by a.

- After k steps back, the input x(n−k) is multiplied by ak.

3. What Correlation Really Does (Intuition)

Think of correlation like sliding one signal over another and checking:

“At which shift do they look most similar?”

If they match well → large positive value
If they do not match → small value
If they are opposite → negative value

1. Definition of Crosscorrelation

The crosscorrelation between two finite-energy signals x(n) and y(n) is:

rxy​(l)=n=−∞∑∞​x(n)y(n−l)

Let me ask you:

When you see y(n−l), what operation is happening to y(n)?
Is it flipping? Or just shifting?

Think carefully.

2. What Does the Lag l Mean?

The parameter l is called the lag.

If:

- l>0 → we shift y(n) to the right

- l<0 → we shift y(n) to the left

So what are we really doing?

We are sliding one signal across the other and measuring similarity at each shift.

Now think:

If the signals match strongly at some shift l=D, what would you expect the value of rxy​(D) to look like? Large or small?

5. Connection to Convolution

You probably noticed something interesting.

Crosscorrelation:

rxy​(l)=∑x(n)y(n−l)

Convolution:

x(n)∗y(n)=∑x(k)y(n−k)

What is missing in correlation compared to convolution?

There is no time reversal (folding).

But the book shows:

rxy​(l)=x(l)∗y(−l)

This means:

Correlation = Convolution with a flipped signal.

So if you already know how to compute convolution, you already know how to compute correlation.

6. Autocorrelation

Now the special case:

rxx​(l)=∑x(n)x(n−l)

Here we compare a signal with itself.

Let me ask you something important:

At l=0, what does rxx​(0) become?

Substitute l=0:

rxx​(0)=∑x(n)2

What is that equal to?

Exactly — the energy of the signal.

So autocorrelation at zero lag equals signal energy.

Now think further:

Would autocorrelation always have its largest value at l=0? Why?